# AI4I 2020 Predictive Maintenance Dataset — Detailed Analysis

Dataset Analysis for the Explainable AI-Driven Digital Twin Project

# 1. What Dataset Are We Actually Using?

The uploaded CSV is the AI4I 2020 Predictive Maintenance Dataset. It contains 10,000 observations and 14 columns. Importantly, this is a synthetic dataset created to reflect predictive-maintenance data encountered in industry. It is not a dataset collected from one named factory, company, or a single identified industrial machine. Instead, it represents a generic manufacturing-machine/process scenario with operating measurements, product variants, tool wear, and machine-failure labels.

Therefore, our project should describe the dataset as a synthetic industrial manufacturing predictive-maintenance dataset, rather than claiming that it represents a particular real-world machine or a specific industrial plant.

Dataset Overview

The uploaded CSV contains 10,000 records and 14 variables. This report profiles its structure, data types, quality, distributions, target behavior, relationships, and suitability for the proposed predictive-maintenance Digital Twin.

# 2. Dataset Context and Industrial Interpretation

Domain: Industrial manufacturing / predictive maintenance.

Dataset type: Synthetic, multivariate, time-series-style machine data.

Records: 10,000.

Columns: 14.

Main purpose: Predict whether a machine/process experiences failure.

Machine representation: Generic production machine/process rather than a named real machine.

Use in our project: Training the predictive model that feeds the Digital Twin.

Numerical variables: 12

Categorical/non-numerical variables: 2

Total missing values: 0

Duplicate rows: 0

Quality interpretation:

No missing values were detected.

No duplicate rows were detected.

# 3. The Meaning of L, M and H

The Type column contains three categories: L, M and H. These do NOT represent three different machines, three industries, or three machine operating modes. They represent product-quality variants.

| Code | Meaning | Approx. Dataset Share | Effect in Dataset |
| --- | --- | --- | --- |
| L | Low quality variant | 50% | Adds 2 min tool wear |
| M | Medium quality variant | 30% | Adds 3 min tool wear |
| H | High quality variant | 20% | Adds 5 min tool wear |

The Product ID begins with L, M, or H and is followed by a serial number. For example, L47181 indicates an L-quality product variant with its own identifier. The quality variant also affects the tool-wear process: the dataset documentation states that H/M/L variants add 5/3/2 minutes of tool wear respectively.

Type

L: 6,000 (60.00%)

M: 2,997 (29.97%)

H: 1,003 (10.03%)

# 4. Complete Feature Interpretation

| Feature | Meaning | Role | Type |  |
| --- | --- | --- | --- | --- |
| UDI | Unique row/observation identifier. | ID | int64 |  |
| Product ID | Product identifier containing quality variant and serial number. | ID / categorical | object |  |
| Type | Product quality variant: L, M, or H. | Categorical feature | object |  |
| Air temperature [K] | Ambient air temperature around the process. | Continuous sensor feature | float64 |  |
| Process temperature [K] | Temperature associated with the machine process. | Continuous sensor feature | float64 |  |
| Rotational speed [rpm] | Machine/tool rotational speed. | Operating feature | int64 |  |
| Torque [Nm] | Torque/load applied during operation. | Operating feature | float64 |  |
| Tool wear [min] | Accumulated tool usage/wear time. | Degradation feature | int64 |  |
| Machine failure | Overall failure indicator: 1 if at least one failure mode occurs. | Primary target | int64 |  |
| TWF | Tool Wear Failure. | Failure-mode target | int64 |  |
| HDF | Heat Dissipation Failure. | Failure-mode target | int64 |  |
| PWF | Power Failure. | Failure-mode target | int64 |  |
| OSF | Overstrain Failure. | Failure-mode target | int64 |  |
| RNF | Random Failure. | Failure-mode target | int64 |  |

# 5. Feature-Level Description

# UDI: numerical; range 1–1e+04; mean 5000; median 5000.

# Product ID: categorical/non-numerical; 10,000 unique values. Frequent values: M14860 (1), L47181 (1), L47182 (1), L47183 (1), L47184 (1).

# Type: categorical/non-numerical; 3 unique values. Frequent values: L (6000), M (2997), H (1003).

# Air temperature [K]: numerical; range 295.3–304.5; mean 300; median 300.1.

# Process temperature [K]: numerical; range 305.7–313.8; mean 310; median 310.1.

# Rotational speed [rpm]: numerical; range 1168–2886; mean 1539; median 1503.

# Torque [Nm]: numerical; range 3.8–76.6; mean 39.99; median 40.1.

# Tool wear [min]: numerical; range 0–253; mean 108; median 108.

# Machine failure: numerical; range 0–1; mean 0.0339; median 0.

# TWF: numerical; range 0–1; mean 0.0046; median 0.

# HDF: numerical; range 0–1; mean 0.0115; median 0.

# PWF: numerical; range 0–1; mean 0.0095; median 0.

# OSF: numerical; range 0–1; mean 0.0098; median 0.

# RNF: numerical; range 0–1; mean 0.0019; median 0.

# 6. Numerical Statistics

| # Feature | # Mean | # Std | # Min | # Median | # Max |
| --- | --- | --- | --- | --- | --- |
| # UDI | # 5000 | # 2887 | # 1 | # 5000 | # 1e+04 |
| # Air temperature [K] | # 300 | # 2 | # 295.3 | # 300.1 | # 304.5 |
| # Process temperature [K] | # 310 | # 1.484 | # 305.7 | # 310.1 | # 313.8 |
| # Rotational speed [rpm] | # 1539 | # 179.3 | # 1168 | # 1503 | # 2886 |
| # Torque [Nm] | # 39.99 | # 9.969 | # 3.8 | # 40.1 | # 76.6 |
| # Tool wear [min] | # 108 | # 63.65 | # 0 | # 108 | # 253 |
| # Machine failure | # 0.0339 | # 0.181 | # 0 | # 0 | # 1 |
| # TWF | # 0.0046 | # 0.06767 | # 0 | # 0 | # 1 |
| # HDF | # 0.0115 | # 0.1066 | # 0 | # 0 | # 1 |
| # PWF | # 0.0095 | # 0.09701 | # 0 | # 0 | # 1 |
| # OSF | # 0.0098 | # 0.09851 | # 0 | # 0 | # 1 |
| # RNF | # 0.0019 | # 0.04355 | # 0 | # 0 | # 1 |

# 7. Target Variable

# Target candidate identified from the uploaded schema: Machine failure.

# 0: 9,661 (96.61%)

# 1: 339 (3.39%)

# 8. Machine Failure and Failure Modes

Machine failure is the main binary target. It is set to 1 when at least one of the five failure modes is true. The five modes represent different mechanisms through which the manufacturing process can fail.

TWF — Tool Wear Failure: Failure associated with excessive tool wear; the dataset models a failure window around 200–240 minutes of tool wear.

HDF — Heat Dissipation Failure: Occurs under a combination of insufficient temperature difference and low rotational speed.

PWF — Power Failure: Occurs when the calculated operating power falls outside the defined acceptable range.

OSF — Overstrain Failure: Occurs when the combination of tool wear and torque exceeds a quality-variant-dependent threshold.

RNF — Random Failure: Represents a small random probability of failure independent of process parameters.

# 9. Important Point About the Dataset

The failure-mode labels are useful for research because they allow the project to go beyond a simple 'failure/no failure' prediction. The model can first predict overall failure risk and the XAI layer can then identify which input variables contributed most strongly to that prediction. However, the UCI documentation notes that the overall machine-failure label does not directly reveal to a machine-learning model which failure mode caused the failure. Therefore, failure-mode columns should be handled carefully to avoid target leakage when designing the primary prediction experiment.

# 10. How L/M/H Fits Our Digital Twin

In our Digital Twin, L/M/H should be treated as product-context information rather than machine identity. A virtual machine state can contain the current product variant, sensor readings, rotational speed, torque, tool wear, and predicted failure risk. The Type value can provide context when interpreting machine behavior and maintenance risk.

# 11. Recommended Project Interpretation

A suitable description for the research proposal is:

“The study uses the AI4I 2020 Predictive Maintenance Dataset, a synthetic industrial manufacturing dataset designed to reflect real-world predictive-maintenance conditions. The dataset represents a generic machine/process environment with temperature, rotational speed, torque, tool-wear, product-quality variants, and multiple machine-failure modes. It is not tied to a particular real-world factory or named machine.”

# 12. Implications for Our Methodology

Use Type as a categorical input after appropriate encoding.

Treat UDI and Product ID primarily as identifiers rather than predictive physical variables.

Use temperature, rotational speed, torque, and tool wear as core machine-state features.

Use Machine failure as the main prediction target.

Consider the individual failure-mode labels for secondary analysis, not as unrestricted inputs to the primary model.

Use SHAP to explain which machine-state variables drive each failure prediction.

Represent the resulting risk and explanation inside the Digital Twin dashboard.

# 13. Dataset Limitations

Synthetic rather than directly collected from a real factory.

No explicit identification of a particular industrial machine or manufacturer.

Static CSV format does not provide a live sensor stream.

The Digital Twin therefore requires a simulated streaming/real-time layer.

Model performance on AI4I should not automatically be generalized to real industrial equipment.

Feature explanations describe model behavior and should not automatically be treated as physical causality.

- 14 Correlation Insights

Air temperature [K] ↔ Process temperature [K]: Pearson r = 0.876

Rotational speed [rpm] ↔ Torque [Nm]: Pearson r = -0.875

Machine failure ↔ HDF: Pearson r = 0.576

Machine failure ↔ OSF: Pearson r = 0.531

Machine failure ↔ PWF: Pearson r = 0.523

Machine failure ↔ TWF: Pearson r = 0.363

UDI ↔ Process temperature [K]: Pearson r = 0.324

Torque [Nm] ↔ Machine failure: Pearson r = 0.191

Torque [Nm] ↔ OSF: Pearson r = 0.183

Tool wear [min] ↔ OSF: Pearson r = 0.156

These associations can guide feature selection and interpretation, but correlation does not establish physical causation.

- 15. Outlier Analysis

UDI: 0 observations outside the 1.5×IQR range.

Air temperature [K]: 0 observations outside the 1.5×IQR range.

Process temperature [K]: 0 observations outside the 1.5×IQR range.

Rotational speed [rpm]: 418 observations outside the 1.5×IQR range.

Torque [Nm]: 69 observations outside the 1.5×IQR range.

Tool wear [min]: 0 observations outside the 1.5×IQR range.

Machine failure: 339 observations outside the 1.5×IQR range.

TWF: 46 observations outside the 1.5×IQR range.

HDF: 115 observations outside the 1.5×IQR range.

PWF: 95 observations outside the 1.5×IQR range.

OSF: 98 observations outside the 1.5×IQR range.

RNF: 19 observations outside the 1.5×IQR range.

In predictive maintenance, outliers may represent genuine abnormal operating states; they should not be removed automatically.

- 16. Key EDA Insights

Sensor and operating variables should be tested for their relationship with the failure target.

Class imbalance should be checked before relying on accuracy as the main metric.

Strongly correlated variables may carry overlapping information and affect XAI feature-attribution results.

Abnormal sensor values may contain useful degradation or failure signals.

Categorical operating modes can provide context for machine-state interpretation.

The dataset can support supervised failure prediction when a validated failure label is available.

- 17. Role in the Proposed Digital Twin

The dataset can represent observations of machine operating state. Its sensor and operating variables form the Digital Twin input layer. A trained ML model can estimate failure risk; SHAP or another XAI method can explain the prediction; and a dashboard can display state, risk, explanations, and rule-based maintenance recommendations.

# 18. Evaluation Metrics

# Accuracy — overall classification correctness.

# Precision — correctness of predicted failures.

# Recall — failures successfully detected.

# F1-score — balance of precision and recall.

# ROC-AUC — class discrimination across thresholds.

# Confusion matrix — false-positive and false-negative analysis.

# 19. Strengths for the Project

# Structured observations suitable for supervised ML.

# Numerical operating measurements support failure-risk modeling.

# Feature-level explanations can be generated with XAI.

# Can serve as the data layer for a Digital Twin prototype.

# Supports reproducible comparison of predictive-maintenance models.

# 20. Dataset-to-Research Mapping

| # Research Component | # Dataset Role | # Output |
| --- | --- | --- |
| # Machine monitoring | # Machine-state features | # Current state |
| # Failure prediction | # Features + target | # Failure probability |
| # Explainable AI | # Model + features | # Feature contributions |
| # Machine health | # Risk + sensor state | # Health/risk score |
| # Maintenance support | # Risk + explanations + rules | # Recommended action |
| # Digital Twin dashboard | # All derived outputs | # Decision support |

# 21. Conclusion

# The uploaded dataset contains 10,000 records across 14 columns and provides the empirical basis for the predictive-maintenance component of the proposed Digital Twin. Its main research value is the connection between machine-state variables and failure behavior. The next modeling stage should explicitly define the target, train/test strategy, imbalance treatment, baseline models, and XAI method so that prediction performance and interpretability can be evaluated together.

# 22. Source

UCI Machine Learning Repository — AI4I 2020 Predictive Maintenance Dataset. DOI: 10.24432/C5HS5C.