UTK-ASL Digital Twin Predictive Maintenance Dataset

Complete Dataset Understanding, Directory Map & Recommended Project Flow

Machine-specific motor testbed • Bently Nevada System1 • Cutsforth InsightCM

EXECUTIVE TAKEAWAY This repository is not one clean ML table. It is a research data archive containing motor-testbed measurements from two monitoring platforms, with two data representations: compact CSV summaries and high-resolution InsightCM JSON event records. For our project, we should NOT use every file. Start with two curated CSVs—CombinedFaults_02222025.csv and final.csv—understand the labels and timestamps, build the baseline machine-state model, and only then open the 24 split JSON files if raw/high-resolution analysis is required.

# 1. What exactly is this dataset?

The UTK-ASL repository is the experimental dataset released with the 2026 paper "Cross-domain digital twin architecture for predictive maintenance via machine learning and Large Language Models." It contains vibration/temperature monitoring data from motor testbeds under healthy baseline and faulty operating conditions. The data are organized around two monitoring platforms: Bently Nevada System1 and Cutsforth InsightCM. The repository documentation reports two motors of the same model and two monitoring platforms, enabling comparative/cross-platform research.

This is machine-specific: the target asset is a motor testbed, not an abstract "factory machine".

It contains physical fault conditions rather than only a generic binary failure flag.

It has time-series information, multiple sensor modalities, and two monitoring-platform representations.

The repository is designed for fault detection, condition monitoring, Digital Twin validation, streaming analytics, and cross-platform comparison.

Important: the seven states are operating/fault conditions, not a guaranteed chronological degradation sequence. Do not interpret them as a natural healthy→failure trajectory.

# 2. What is inside the repository?

Dataset_digital_twin_predictive_maintenance/
├── README.md
├── data/
│   ├── BentlyNevada_System1/
│   │   ├── README.md
│   │   ├── s1_testbed_data.csv
│   │   ├── CombinedFaults_02222025.csv
│   │   ├── 01232025_Baseline/
│   │   ├── 01302025_Baseline/
│   │   ├── 01222025_EccentricRotor/
│   │   ├── 02052025_BentShaft/
│   │   ├── 02132025_FaultedBearing/
│   │   ├── 02172025_FaultedCoupling/
│   │   ├── 02172025_Imbalance/
│   │   └── CombinedFiles/
│   │       ├── Combined_B1.csv
│   │       ├── Combined_B2.csv
│   │       ├── Combined_EC.csv
│   │       ├── Combined_FB.csv
│   │       ├── Combined_FC.csv
│   │       ├── Combined_BS.csv
│   │       └── Combined_IM.csv
│   └── Cutsforth_InsightCM/
│       ├── README.md
│       ├── final.csv
│       ├── icm_chiller.json
│       ├── icm_chiller_split_10mb/
│       │   ├── icm_chiller_part001.json
│       │   ├── ...
│       │   └── icm_chiller_part024.json
│       └── image/
├── paper/
│   ├── 1-s2.0-S0360835226001154-main.pdf
│   └── readme.md
└── LICENSE

# 3. Directory-by-directory explanation

| Path | What it means | Our use |
| --- | --- | --- |
| README.md | Repository-level guide; explains the dataset, intended uses, structure, quick-start loading, reconstruction, limitations, and citation. | Read once; keep as reference. |
| data/ | Main data area. | Everything used for modeling/analysis comes from here. |
| BentlyNevada_System1/ | CSV-based monitoring data from the Bently Nevada System1 platform. | Use for compact multi-fault analysis and a strong baseline. |
| Cutsforth_InsightCM/ | Higher-resolution multi-sensor monitoring data from InsightCM. | Use for dense time-series, sensor fusion, and streaming-style experiments. |
| paper/ | Published research paper and notes. | Use to understand the original experiment and avoid accidentally reproducing the published contribution. |
| LICENSE | Repository license information. | Check before redistribution; repository states MIT licensing. |

# 4. Bently Nevada System1 — complete map

System1 is the easier side of the repository. The official README describes approximately 15,000 records, two accelerometer channels, roughly one-minute sampling, and six metrics per channel: bias, derived peak, direct, RMS, velocity peak, and velocity RMS.

| Item | One-line explanation | Use? |
| --- | --- | --- |
| s1_testbed_data.csv | Testbed/configuration summary; describes the equipment/setup rather than being the primary ML table. | YES — documentation |
| CombinedFaults_02222025.csv | Master System1 CSV combining the fault conditions into one analysis-ready file. | YES — primary System1 file |
| 01232025_Baseline/ | First healthy/baseline measurement run. | YES — healthy reference |
| 01302025_Baseline/ | Second healthy/baseline measurement run. | YES — baseline variability |
| 01222025_EccentricRotor/ | Experiment associated with eccentric-rotor fault condition. | YES — fault class |
| 02052025_BentShaft/ | Experiment associated with bent-shaft fault condition. | YES — fault class |
| 02132025_FaultedBearing/ | Experiment associated with bearing fault condition. | YES — fault class |
| 02172025_FaultedCoupling/ | Experiment associated with coupling/misalignment fault condition. | YES — fault class |
| 02172025_Imbalance/ | Experiment associated with rotor-imbalance condition. | YES — fault class |
| CombinedFiles/ | Pre-aggregated per-state CSV files, one for each baseline/fault condition. | OPTIONAL — useful for state-wise analysis |
| Combined_B1.csv | Combined Baseline 1 measurements. | Optional |
| Combined_B2.csv | Combined Baseline 2 measurements. | Optional |
| Combined_EC.csv | Combined eccentric-rotor measurements. | Optional |
| Combined_FB.csv | Combined faulted-bearing measurements. | Optional |
| Combined_FC.csv | Combined faulted-coupling measurements. | Optional |
| Combined_BS.csv | Combined bent-shaft measurements. | Optional |
| Combined_IM.csv | Combined imbalance measurements. | Optional |

# 5. The seven System1 states

| State | Meaning | Role in our Digital Twin |
| --- | --- | --- |
| Baseline 1 | Healthy reference run. | Normal-state calibration. |
| Baseline 2 | Second healthy reference run. | Check normal-condition variability. |
| Eccentric Rotor | Rotor operates with eccentricity. | Fault diagnosis / signature learning. |
| Bent Shaft | Shaft is bent. | Fault diagnosis / signature learning. |
| Faulted Bearing | Bearing fault condition. | Fault diagnosis / maintenance reasoning. |
| Faulted Coupling | Coupling/misalignment condition. | Fault diagnosis / maintenance reasoning. |
| Imbalance | Rotor imbalance condition. | Fault diagnosis / maintenance reasoning. |

# 6. Cutsforth InsightCM — complete map

InsightCM is the more confusing side because the same underlying measurements are represented as a processed CSV and as high-resolution JSON records. The repository reports 852,860 discrete sensor readings, 4 accelerometers, 4 thermocouples, and 1 tachometer, with approximately 1–5 second temporal resolution.

| Item | One-line explanation | Our use |
| --- | --- | --- |
| final.csv | Processed/summary CSV recommended by the repository for most analysis; much easier than raw JSON. | PRIMARY InsightCM file |
| icm_chiller.json | Single reconstructed high-resolution JSON representation of the InsightCM measurements; about 244 MB. | Do not start here; use only when raw records are required. |
| icm_chiller_split_10mb/ | Directory containing the large JSON split into 24 roughly 10-MB chunks for Git-friendly distribution. | Use only for raw/high-resolution work. |
| icm_chiller_part001.json … part024.json | Chunks of the same large JSON dataset; NOT 24 separate experiments. | Concatenate/read sequentially only if needed. |
| image/ | Supporting images/figures associated with the dataset. | Reference only; not a training source. |

# 7. What are the JSON files actually doing?

The JSON is a storage representation of the InsightCM event/sensor records. The 24 part files are simply a Git-friendly split of one large data collection. They should not be interpreted as 24 machines, 24 fault types, or 24 experiments.

ONE LARGE INSIGHTCM DATASET
        │
        ├── part001.json
        ├── part002.json
        ├── ...
        └── part024.json
              │
              ▼
       combined sensor records
              │
              ▼
        pandas DataFrame
              │
              ▼
     time-series analysis

The repository also provides a reconstruction approach: load parts 001–024, extend the record list, and optionally write one reconstructed JSON file. For most project work, however, the repository explicitly recommends using final.csv instead.

# 8. CSV vs JSON — which should we use?

| Representation | Strength | Weakness | Decision |
| --- | --- | --- | --- |
| System1 CombinedFaults_02222025.csv | Compact, labeled, easy to model, fault-oriented. | Lower temporal resolution; fewer sensor modalities. | START HERE for baseline/fault classifier. |
| System1 CombinedFiles/*.csv | Convenient state-by-state analysis. | Duplicates the information in the master/combined view. | Use selectively. |
| InsightCM final.csv | Small, processed, easier multi-sensor analysis. | May not retain every raw event detail. | START HERE for InsightCM. |
| InsightCM split JSONs | High-resolution/raw event representation; richest source for custom temporal processing. | Large, nested, harder to parse; easy to misuse. | Use later, only if needed. |
| icm_chiller.json | One-file raw/high-resolution representation. | Large (~244 MB), unnecessary for initial work. | Usually ignore initially. |

# 9. The most important conceptual distinction

DO NOT THINK: 24 JSON files = 24 datasets. THINK: 24 JSON files = one large InsightCM record set split into manageable chunks. DO NOT THINK: Baseline → eccentric rotor → bent shaft → bearing → coupling → imbalance is a degradation timeline. THINK: these are separate labeled operating/fault conditions. They are excellent for diagnosis/classification, but they do not by themselves establish a true chronological RUL trajectory.

# 10. Recommended project approach — the clean flow

PHASE 0 — REPOSITORY UNDERSTANDING
        ↓
Read README + platform READMEs + paper
        ↓
PHASE 1 — DATA AUDIT
        ↓
Load ONLY:
  • System1/CombinedFaults_02222025.csv
  • InsightCM/final.csv
        ↓
Inspect columns, timestamps, labels, missingness, sampling intervals
        ↓
PHASE 2 — LABEL & SENSOR MAPPING
        ↓
Map:
  sensor → physical quantity → platform → timestamp → state/fault
        ↓
PHASE 3 — HEALTHY BASELINE
        ↓
Use Baseline 1 + Baseline 2 to learn normal machine behaviour
        ↓
PHASE 4 — FEATURE ENGINEERING
        ↓
Vibration: RMS, peak, std, kurtosis, skewness, crest factor, peak-to-peak
Temperature: mean, trend, variance, rate of change
Speed: mean, variance, stability
Cross-sensor: correlations, vibration magnitude, temp-vibration coupling
        ↓
PHASE 5 — BASELINE ML
        ↓
Random Forest / XGBoost / SVM
Task: 7-state fault diagnosis
        ↓
PHASE 6 — DIGITAL TWIN STATE
        ↓
Current machine state + health index + anomaly score + confidence
        ↓
PHASE 7 — EXPLAINABILITY
        ↓
SHAP / feature contribution
"What sensor behaviour caused this diagnosis?"
        ↓
PHASE 8 — CROSS-PLATFORM VALIDATION
        ↓
Compare / transfer / validate between System1 and InsightCM
        ↓
PHASE 9 — WHAT-IF SIMULATION
        ↓
"What happens if vibration rises 15%?"
"What happens if temperature increases?"
        ↓
PHASE 10 — MAINTENANCE DECISION SUPPORT
        ↓
Healthy / Monitor / Inspect / Schedule maintenance
        ↓
PHASE 11 — DASHBOARD
        ↓
Digital Twin view:
  live/simulated sensor state → prediction → explanation → action

# 11. What should be our primary research problem?

The strongest use of this dataset is not simply "predict machine failure." A better framing is: build a machine-specific Digital Twin state representation that fuses vibration, temperature and rotational information, diagnoses known motor fault states, quantifies machine health/anomaly level, explains the diagnosis, and evaluates robustness across two monitoring platforms.

| Component | Proposed implementation | Why it matters |
| --- | --- | --- |
| Machine state | 7-class state representation with healthy baselines and five faults. | Represents a specific motor testbed rather than a generic machine. |
| Health index | 0–100 composite health score calibrated from normal-state distance + fault probability. | Provides a continuous Digital Twin state variable; clearly document that it is a derived index, not a measured physical quantity. |
| Anomaly score | Distance from healthy baseline in feature space or reconstruction error. | Supports early warning before hard classification. |
| Explainability | SHAP on the classifier; feature-level explanations. | Turns a prediction into maintenance-relevant evidence. |
| Cross-platform validation | Compare predictions/feature behaviour across System1 and InsightCM after careful normalization/alignment. | A strong research angle enabled by this repository. |
| What-if layer | Perturb selected sensor features within physically plausible ranges and recompute model state. | Moves the project toward decision support rather than pure classification. |
| Dashboard | Sensor state + fault + health + explanation + maintenance action. | Makes the Digital Twin operationally understandable. |

# 12. Exact workflow we should follow in implementation

Clone/download the repository and preserve the original directory structure as read-only raw data.

Create a separate project data layer. Never edit raw files directly.

Load CombinedFaults_02222025.csv and final.csv into separate DataFrames.

Print shape, column names, data types, timestamp ranges, unique labels, missing-value counts, duplicate timestamps, and sampling intervals.

Create a data dictionary mapping every useful column to sensor type, physical meaning, unit (when documented), platform, and role.

Normalize timestamps to a single timezone/format without inventing synchronization where it does not exist.

Build the System1 baseline first. This gives the team a simple, auditable fault-classification pipeline.

Build the InsightCM pipeline second. Preserve its time-series nature and test window-based features.

Use the two baseline runs to quantify normal-condition variability before defining health thresholds.

Train a baseline classifier with a leakage-safe split. Do NOT randomly split adjacent time samples if that would put nearly identical windows in train and test.

Evaluate using macro-F1, per-class precision/recall, confusion matrix, balanced accuracy and ROC-AUC where appropriate—not accuracy alone.

Add SHAP explanations and verify that the dominant features are physically plausible.

Construct the Digital Twin state layer from sensor state + diagnosis + health/anomaly score + confidence + timestamp.

Only after both pipelines work independently, attempt cross-platform alignment/validation.

Only use the raw JSON chunks if the processed CSV cannot answer a research question.

Build the dashboard last, after the analytical pipeline is stable.

# 13. Data leakage and validation rules

Do not randomly split highly adjacent time-series rows into train/test; this can inflate performance.

Prefer run-wise, time-wise, or condition-aware splits where possible.

Do not train on the same experimental run and claim generalization to a new run without a clear protocol.

Do not treat multiple JSON chunks as independent samples; they are pieces of the same underlying collection.

Do not compare raw System1 and InsightCM numerical values as if their sensors/processing chains were identical. Normalize and document platform differences first.

Do not call a model probability a physical health measurement. A health index must be explicitly defined as a derived indicator.

Do not claim RUL unless the data provide a defensible degradation trajectory and failure-time target.

# 14. What we should NOT use initially

| File / Folder | Why not initially? |
| --- | --- |
| icm_chiller.json | Large raw representation; unnecessary until we prove final.csv is insufficient. |
| icm_chiller_split_10mb/*.json | Same reason; 24 chunks are one underlying record set, not separate datasets. |
| image/ | Supporting figures, not model input. |
| Every dated System1 folder simultaneously | Creates unnecessary preprocessing complexity; use master combined CSV first. |
| Every CombinedFiles CSV simultaneously | Mostly redundant for the first pipeline; use them for state-specific inspection when needed. |

# 15. What questions this dataset can answer

| Question | Can dataset support it? | Comment |
| --- | --- | --- |
| Which motor fault is present? | YES | Strong classification use case. |
| What sensor behaviour distinguishes faults? | YES | Excellent for feature analysis/XAI. |
| What is normal motor behaviour? | YES | Two baseline measurements help quantify baseline variation. |
| Can vibration + temperature + speed improve diagnosis? | YES | Especially with InsightCM. |
| Can we build a machine-specific Digital Twin state? | YES | Strong conceptual fit. |
| Can we compare two monitoring platforms? | YES | One of the dataset's intended uses. |
| Can we do real-time/streaming simulation? | YES | High-resolution InsightCM records support this. |
| Can we prove true RUL? | NOT DIRECTLY | No clear run-to-failure degradation trajectory across these labeled states. |
| Can we claim universal machine generalization? | NO | Single testbed / limited conditions. |
| Can we simulate arbitrary physical operating conditions? | LIMITED | Only data-supported perturbations should be used unless a physics model is added. |

# 16. Recommended final system architecture

┌──────────────────────────────┐
                 │      MOTOR TESTBED DATA       │
                 └──────────────┬───────────────┘
                                │
              ┌─────────────────┴─────────────────┐
              │                                   │
      Bently Nevada                         Cutsforth InsightCM
        System1                                  │
              │                           vibration + temp + RPM
              │                                   │
              └─────────────────┬─────────────────┘
                                ↓
                     DATA INGESTION LAYER
                                ↓
                    CLEANING + SYNCHRONIZATION
                                ↓
                     SENSOR/FEATURE LAYER
                                ↓
             ┌────────────────┼────────────────┐
             ↓                ↓                ↓
        Fault Model       Health Index      Anomaly Model
             │                │                │
             └────────────────┼────────────────┘
                              ↓
                    DIGITAL TWIN STATE
                              ↓
                     EXPLAINABLE AI
                              ↓
                    WHAT-IF SIMULATION
                              ↓
                MAINTENANCE DECISION SUPPORT
                              ↓
                         DASHBOARD

# 17. Proposed project modules

| Module | Input | Output |
| --- | --- | --- |
| M1 — Data ingestion | Raw/curated CSVs | Standardized tables |
| M2 — Data audit | Standardized tables | Data quality report |
| M3 — Sensor mapping | Columns + platform docs | Machine data dictionary |
| M4 — Feature engineering | Sensor time series | Feature matrix/windows |
| M5 — Fault diagnosis | Features | Fault class + probability |
| M6 — Anomaly detection | Healthy baseline + windows | Anomaly score |
| M7 — Health index | Diagnosis + anomaly + sensor state | 0–100 derived health indicator |
| M8 — Explainability | Trained model | SHAP/global/local explanations |
| M9 — Cross-platform validation | System1 + InsightCM | Robustness/comparison results |
| M10 — What-if engine | Current feature state | Counterfactual predictions |
| M11 — Decision support | Twin state + explanations | Maintenance recommendation |
| M12 — Dashboard | All outputs | Interactive Digital Twin UI |

# 18. A practical folder structure for OUR project

digital_twin_motor/
│
├── data/
│   ├── raw/                  # untouched downloaded repository
│   ├── interim/              # cleaned/intermediate files
│   └── processed/            # modeling-ready datasets
│
├── notebooks/
│   ├── 01_repository_audit.ipynb
│   ├── 02_system1_eda.ipynb
│   ├── 03_insightcm_eda.ipynb
│   ├── 04_feature_engineering.ipynb
│   ├── 05_fault_model.ipynb
│   ├── 06_health_anomaly.ipynb
│   ├── 07_xai.ipynb
│   └── 08_cross_platform.ipynb
│
├── src/
│   ├── data_loader.py
│   ├── preprocessing.py
│   ├── features.py
│   ├── models.py
│   ├── health_index.py
│   ├── explainability.py
│   ├── what_if.py
│   └── evaluation.py
│
├── dashboard/
│   └── app.py
│
├── models/
├── reports/
└── README.md

# 19. Final recommendation — how we should handle the confusing repository

USE NOW → BentlyNevada_System1/CombinedFaults_02222025.csv + Cutsforth_InsightCM/final.csv USE FOR REFERENCE → s1_testbed_data.csv + platform READMEs + paper USE SELECTIVELY → CombinedFiles/*.csv for fault-by-fault inspection USE LATER → icm_chiller_split_10mb/*.json when we need raw/high-resolution records IGNORE INITIALLY → icm_chiller.json and image/ for the first pipeline Recommended order: System1 baseline → InsightCM time-series → sensor/feature mapping → fault diagnosis → health/anomaly layer → SHAP → cross-platform validation → what-if simulation → maintenance decision support → dashboard.

# 20. Source and citation notes

Primary source: UTK-ASL, Dataset: Digital Twin Predictive Maintenance, repository README and platform documentation. The repository states that the dataset supports the 2026 Computers & Industrial Engineering paper by Williams et al. The repository also provides the detailed directory structure, data overview, recommended CSV entry points, JSON reconstruction method, and limitations.

Repository: https://github.com/UTK-ASL/Dataset_digital_twin_predictive_maintenance

Paper: https://doi.org/10.1016/j.cie.2026.111914

Important published baseline: The 2026 paper already presents a cross-domain Digital Twin architecture using ML/LLM components. Our implementation should therefore be framed as an extension/research variant, not a reproduction.